WAT-TN-001

Technical Note regarding uC/USB & uC/FS Bugs IN PERCIVAL SYSTEM

Author: HUGO GATEAU

Approved by: Luc Filion

Release Version: 0.1

DATE: 2016-08-31

CHANGE  HISTORY

| VERSION | DATE | ADM | CHANGE DESCRIPTION (W/ SECTIONS) | CR NUMBER |
| --- | --- | --- | --- | --- |
| Rev 0.1 | 2018-08-31 | A | ORIGINAL DOCUMENT | N/A |

*A – ADDED   M – MODIFIED  D – DELETED

# Setup

| IDE | IAR Workbench for ARM 6.5 |
| --- | --- |
| MCU | STM32F407IGH6 |
| Hardware Platform | Watlow Percival board |
| Hardware peripheral | USB stick SD card Nexus 7 Tablet with Android 6.0.1 (Kernel 3.4.1-g2ab7501) Watlow EZ-ZONE® RM (2 units) Ellisys USB Sniffer EX-200 |
| Micrium related product | uC/OS-II version 2.92 uC/USB-Host-V3 version 3.41 uC/LIB 1.37.02 uC/HTTPs 2.0 uC/FS version 4.5 except for sd device( 4.6) and msc device (4.7). |
| Software | Android Open Accessory protocol version 2 FAT32 |

Figure 1 : Picture of the test setup

# Team at work

Hugo Gateau, Christian Mamen, Ahmed Faiz, Luc Filion

# Problem description

The Percival system has three basic memories that use the Micrium file system, NOR flash, SD card and USB stick.  The NOR flash is used to store the nonvolatile parameters.  The SD card contains the web pages and is used to store data logging data.

File system is FAT32 on both devices. Sometimes, data are unexpectedly mixed between memories (file from website are transferred to USB stick). Therefore, the file index becomes corrupted, the device is unreadable, files are erased, and so on…

The issue is not easily reproducible but two factors are determinant: CPU load which is high and memory access which are numerous and simultaneous. After investigation, communications with the Android device generates a lot of interruptions which cause CPU load to be very high.

## Problem 1: Too many NAK

### USB communication description

In the USB communication, the Percival board is the host and the Android tablet is the device. The host initiates all transfers.

In the application, the host executes in a loop sequence:

- read data from device;

- send data to device;

- wait 5 ms.

### Issue description

To initiate the transfer, the USB host sends a packet to the device.

When the device has no data or when the USB controller cannot respond to the host (for whatsoever reason), it replies with a NAK.

After a NAK is received by the host, the host resends the same packet until it gets the data.

This is a normal behavior for a USB communication with a bulk endpoint. Bulk endpoints are required by the Android Open Accessory protocol so we cannot change the type of transfer.

### Technical Explanation

When a NAK is received, an interrupt is triggered. The interrupt disables the channel and re-enable it. This forces the USB controller to resend the channel last transmitted request. This is a very aggressive resend strategy and it can cause up to 125 retry per ms.

### Implemented Solution

Instead of responding to NAK immediately in the ISR as it was, we process them in a separate resend queue. A timer is used and asynchronously do the resend every 100 us for all channel in the queue. After several tests, the value of 100 us has been identified as optimal. We observe a reduction of NAK of a factor 10.

NOTE 1: The timer period and the size of the queue could be adjusted, if required. Minimally, the size of the queue should be the same size as the number of USB channels.

NOTE 2: There are two types of requests

- IN (Host asking the Device to send some data)

- OUT (Host send data to device).

The fix has been applied only for IN requests.

Future upgrades or modifications to the USB system may require updating and modifying the timer and queuing size.

## Problem 2: File System

Reducing the interrupt frequency caused by too many USB NAK received reduced the CPU load as expected. But it did not fix the data corruption issue.

### Setup used to reproduce the issue

In order to reproduce the issue we need to generate a lot of traffic on both SD card and USB stick. This can be done by simultaneously refreshing the website continuously (every second for example) and having a data transfer running (transferring the log from …).

In order to refresh the website we use an AutoIt script that simulates a F5 keystroke of the browser every one second.

Using this setup usually takes less than one hour to see a manifestation of the problem above mentioned.

### Bug tracking and Technical explanation

To track the bug and reproduce it more frequently, we modified the application code so that we wrote a specific pattern to the SD card when logging data. Then, during data transfer to USB stick, we checked that the data were corresponding to the pattern we previously set and finally we tried to find discrepancies.

At the low level of the a FAT32 file system, we can only read and write sectors in whole (smallest write unit) . So when the system needs to write less than a full sector, the FAT file system module does the following:

- Read the sector N,

- Save the sector in a temporary buffer,

- Modify  the data in the buffer,

- Rewrite buffer to sector N.

Therefore, the corruption issues observed may be caused by a single byte to write but affecting a whole sector.

These temporary buffers described in (3) above are sometimes (but extremely rarely) overwritten by another file write request happening from another device. This basically causes that the same buffer is in use by two different devices at the same time. That explains SD card data in the USB stick. As this behavior could happen anytime for any given memory location and file, this explains file deletion and file system corruption in general.

All buffers are obtained by using memory pool. A memory pool is an array of buffer allocated statically but handled dynamically by using Mem_PoolBlkGet and Mem_PoolBlkFree from the uc_lib service. This buffer is used by the File System (and not Watlow application).

Mem_PoolBlkFree showed a bug that allowed the pool to be modified while the release of a block was not completed. This can lead to a buffer overwriting.

### Bug fix

To avoid this behavior to happen, we expanded the critical section to the whole function Mem_PoolBlkFree, so that interruption of the memory buffer deallocation is not allowed.

This fix does not decrease the overall performance but it can increase latency for some tasks compare to prior but because Mem_PoolBlkFree is a relatively short function with only basic operations we do believe that the impact will be minimal.

# Recommendations

## Mismatches in module files

We noticed that some Micrium modules in your project show some mixed versions files and/or mismatches and should be fixed (at a minimum, bring every files of a same module to the same version).

For example uC/OS-II file have version from 2.92.09 to 2.92.11 and uC/FS from version 4.5.3 to 4.7.

## Upcoming Deprecation

Also we would like to inform you that starting from version 1.38, Memory heap allocation and static memory pool features are deprecated and are expected to be removed in a future version of µC/LIB. This is important because a lot of Micrium modules are dependent of µC/LIB. In particular next versions of the file system will no longer be compatible with your current version of µC/LIB.

# Appendix

## USB Read without patch

## USB read operation with the patch: